SIMULATION TOOL CISCO PACKET TRACER

Experiment: Installation and Basic Network Configuration Using Cisco Packet Tracer

# AIM:

To install Cisco Packet Tracer and create a simple network consisting of two computers connected through a switch, configure IP addresses, and verify connectivity.

# REQUIREMENTS:

Computer/Laptop, Internet connection, Cisco Packet Tracer software, and a Cisco Networking Academy or Skills for All account.

# INTRODUCTION:

Cisco Packet Tracer is a network simulation software used to design and test computer networks. It provides virtual routers, switches, computers, cables and other networking devices, allowing network configurations to be tested without physical hardware.

# INSTALLATION PROCEDURE:

Step 1: Open the Cisco Networking Academy or Skills for All website and sign in with your account.

Step 2: Go to the Packet Tracer download section and select the installer suitable for your operating system.

Step 3: After the download is complete, locate the installer and double-click it to start the installation.

Step 4: Read the License Agreement, select the option to accept the agreement, and click Next.

Step 5: Select the installation folder. The default folder can be used unless another location is required.

Step 6: Select the required shortcuts and click Next.

Step 7: Click Install and wait until the installation is completed.

Step 8: Click Finish and open Cisco Packet Tracer.

Step 9: Sign in using the required Cisco account. Packet Tracer is now ready to use.

# PROCEDURE FOR BASIC NETWORK SETUP

- Open Cisco Packet Tracer.

- From End Devices, place two PCs in the workspace.

- From Network Devices → Switches, place one switch.

- Select the Copper Straight-Through cable and connect PC0 to the switch.

- Connect PC1 to another port of the switch using the same cable.

- Open PC0 → Desktop → IP Configuration and enter IP address 192.168.1.10 with subnet mask 255.255.255.0.

- Open PC1 → Desktop → IP Configuration and enter IP address 192.168.1.11 with subnet mask 255.255.255.0.

- Open PC0 → Desktop → Command Prompt and type ping 192.168.1.11

# NETWORK CONFIGURATION

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.11 | 255.255.255.0 |

# RESULT

Cisco Packet Tracer was installed successfully. A basic LAN was created using two PCs and a switch, and connectivity between the PCs was successfully verified using the ping command.